# AV Device Security Guidelines

Document Version: 1.0
Prepared by: OpenAVCloud Technical Working Group
Date: October 2, 2025

# Table of Contents

Table of Contents	2

Document History and Change Log	3

1.	Purpose	4

2.	Security Requirements	5

2.1	Firmware Integrity and Secure Boot	5

2.2	Secure Communications (Data in Transit)	5

2.3	Network Authentication and Access Control	5

2.4	Secure Storage (Data at Rest)	5

2.1	Device Provisioning and Identity	6

2.2	Lifecycle Security and Vulnerability Management	6

3.	Additional Considerations	7

3.1	Password Management	7

3.2	Secure Development and Supply Chain Practices	7

3.3	Incident Response and Logging	7

3.4	Application Security	7

4.	Compliance and Auditing	7

4.1	Security Audits	7

5. References	8

# Document History and Change Log

This section captures the history of changes made to this document.

| Version | Date | Reason for Change |
| --- | --- | --- |
| 1.0 | 2025-10-02 | Initial release of AV Device Security Guidelines. |

# Purpose

This document outlines the minimum security guidelines required for Audio/Video (AV) devices developed, deployed, or integrated by members of the OpenAVCloud initiative.

# Security Requirements

# Firmware Integrity and Secure Boot

    - Devices must implement an irrevocable hardware Secure Boot process.

    - Secure Boot must be enabled by default.

    - Devices must prevent unauthorized and unauthenticated software from being loaded. If permitted, it must run in a sandboxed or limited-permission environment.

    - Remote software updates must be digitally signed by a trusted authority.

    - Devices must verify the digital signature and certificate chain before updates.

# Secure Communications (Data in Transit)

    - Devices must use certificate pinning or equivalent for TCP/IP or UDP/IP communication.

    - TCP protocols (e.g., MQTT) must be protected by TLS.

    - UDP protocols (e.g., CoAP) must be protected by DTLS.

    - Cryptographic suites must be validated against NIST 800-131A or OWASP.  Unsecure suites must be removed.

# Network Authentication and Access Control

    - Devices must support a secure network authentication method such as 802.1X.

    - All unused ports must be closed by default.

    - Treat auto-discovery conservatively by default and restrict its scope to intended domains and purposes

    - Debug interfaces must be disabled or protected via a best practice authentication or access control mechanism.

    - Debug interfaces that are physical ports should be physically protected by the device.

    - Resilience should be built into the device, taking into account the possibility of outages of data networks and power.

# Secure Storage (Data at Rest)

    - Devices must not contain hardcoded credentials.

    - Network communication keys must be stored securely.

    - Passwords must be stored using industry-standard cryptographic algorithms.

    - Users must be provided with functionality such that all their user data can be erased from the consumer IoT device in a simple manner (factory reset).

# Device Provisioning and Identity

    - Devices must have a unique and tamper-resistant identifier.

    - Secure provisioning must include unique generation, distribution, update, revocation, and destruction of keys.

# Lifecycle Security and Vulnerability Management

    - Vulnerability assessments must be conducted before launch and periodically thereafter.

    - The manufacturer must make a vulnerability disclosure policy publicly available. This policy must include, at a minimum:

      - contact information for the reporting of issues; and

      - timelines for initial acknowledgement of receipt of a vulnerability report; and

      - timelines for when the person who reported the issue will receive status updates until the resolution of the reported issues.

    - The manufacturer must publish, in an accessible way that is clear and transparent to the user, the defined support period.

# Additional Considerations

# Password Management

    - Enforce strong password policies including complexity, rotation, and secure recovery.

    - Avoid default passwords and ensure initial credentials are changed at first use

    - Establish mutual trust for device-to-device and device-to-gateway communications, with credentials managed for expiration and revocation

# Secure Development and Supply Chain Practices

    - Follow secure coding practices, code signing, and vulnerability scanning.

    - Ensure vendors comply with supply chain security standards including SBOM transparency.

# Incident Response and Logging

    - Support event logging for security actions.

    - Maintain a documented incident response plan including detection, containment, and remediation.

# Application Security

    - All applications running on the device, including third-party apps, must follow secure development practices.

# Compliance and Auditing

# Security Audits

    - All AV devices must undergo regular security audits and demonstrate compliance with this standard.

# 5. References

- NIST SP 800-131A

- OWASP IoT Top 10

- ISO/IEC 27001

- IEEE 802.1X